| Sr. No. | Test Parameter | Specification |
| --- | --- | --- |
| 01 | #### Description | White, practically odorless, crystalline powder. |
| 02 | Solubility | Slightly soluble in water and in methanol; insoluble in benzene, in carbon tetrachloride, and in chloroform. |
| 03 | Identification - By IR - By HPLC | The infrared absorption spectrum of the sample is concordant with the spectrum obtained with Amoxicillin Trihydrate or with the reference spectrum of Amoxicillin Trihydrate. The retention time of the major peak of the sample solution corresponds to that of the Standard solution, as obtained in the Assay. |
| 04 | Organic Impurities | By HPLC. |
|  | #### Amoxicillin related compound Ia (D-hydroxyphenylglycine) | Not more than 1.0% |
|  | #### Amoxicillin related compound Db,c (Amoxicillin open ring) | Not more than 1.0% (RRT : 0.53) Not more than 1.0% (RRT : 0.68) |
|  | #### Amoxicillin related compound Ad (6-aminopenicillanic acid) | Not more than 0.5% |
|  | #### Amoxicillin related compound Gg (D- hydroxyphenylglycyl-amoxicillin) | Not more than 1.0% |
|  | #### Amoxicillin related compound Eh (amoxicillin penilloic derivative) | Not more than 1.0% |
|  | #### Amoxicillin related compound Mi (N-(penicillan-6-yl) open ring amoxicillinamide) | Not more than 1.0% |
|  | #### Amoxicillin related compound Cl (amoxicillin rearrangement product) | Not more than 1.0% |
|  | #### Amoxicillin related compound Eh,i (amoxicillin penilloic derivative) | Not more than 1.0% |
|  | #### Amoxicillin related compound Jm (amoxicillin open ring dimer) | Not more than 1.0% |
|  | #### Amoxicillin related compound Ln #### (N-(penicillan-6-yl) amoxicillinamide) | Not more than 1.0% |
|  | #### Any unspecified individual impurity | Not more than 1.0% |
|  | #### Total Impurities | Not more than 5.0% |
| 05 | Crystallinity | Particle shows birefringence and extinction position when microscope stage revolved. |
| 06 | Dimethylaniline | Not more than 0.002% (20 ppm). |
| 07 | pH | Between 3.5 and 6.0 |
| 08 | Water Determination | Between 11.5 and 14.5% |
| 09 | Assay (By HPLC) | Not less than 900 µg & not more than 1050 µg/mg of Amoxicillin (C16H19N3O5S), on anhydrous basis. |