| Sr. No. | Test Parameter | Specification |
| --- | --- | --- |
| 1. | Description | Clear colorless solution filled in transparent glass vial with transparent flip off seal (Refer the manufacturer specification) |
| 2. | Identification : |  |
|  | A) Granisetron Hydrochloride | By HPLC; The UV Spectrum of major peak of the sample solution corresponds to that of the standard solution, as obtained in the Assay. |
|  | B) Benzyl Alcohol | By HPLC; The retention time of major peak of the sample solution correspond to that of the standard solution, as obtained in the Assay. |
| 3. | pH | Between 4.00 and 6.00 |
| 4. | Average Volume | Not less than 3.00 ml. |
| 5. | Uniformity of Volume | 3.00 to 3.45 ml. |
| 6. | Bacterial Endotoxins | Not more than 25 USP Endotoxin Units per mg of Granisetron. |
| 7. | Sterility | Sample should be sterile. |
| 8. | Particulate Matter : |  |
|  | a) ≥ 10 µm | Not more then 6,000 particles / container. |
|  | b) ≥ 25 µm | Not more then 600 particles / container |
| 9. | Related Compounds - By HPLC |  |
|  | Granisetron Related Compound C | Not more than 0.7% |
|  | Total Specified Impurities | Not more than 1.3% |
|  | Any Unspecified Impurity | Not more than 0.5% |
| 10. | Assay |  |
|  | Each ml contains : Granisetron Hydrochloride USP eq. to Granisetron 1 mg | 0.93 to 1.07 mg (93.00 to 107.00% of labeled amount) |